AC CURRENT SWTICH

CS-610-200 Series

The CS-610-200 series incorporates:

- auto-ranging

- self-powered and no insertion loss

- true digital switching and no leakage

- field adjustable setpoint

- status and power LED’s

- solid-state reliability

- input / output isolation via current transformer

PRODUCT HIGHLIGHTS

- compact form factor

- solid, reliable mounting

- integrated din rail mounting

SPECIFICATIONS

| SPECIFICATIONS |  |
| --- | --- |
| CURRENT SETPOINT | Adjustable - 1 to 200 Amps |
| MAXIMUM INPUT CURRENT | 200 Amps continuous |
| SENSOR POWER | Self-powered |
| OUTPUT TYPE | Solid-state mosfet |
| OUTPUT SWITCH ACTION | Normally open |
| OUTPUT SWITCH RATINGS | 30 Vac/dc, 500 mA max |
| VON @ 24 VDC AT 500 mA | <50 mV |
| FREQUENCY | 50/60 Hz |
| RESPONSE TIME | 200 mS typical |
| INSULATION CLASS | 600 Vac, insulated conductors |
| OPERATING TEMPERATURE | -15 to 50°C (5 to 122°F) |
| OPERATING HUMIDITY | 5 to 90 %RH non-condensing |
| WIRING CONNECTIONS | Screw terminal block (14 to 22 AWG) |
| DIMENSIONS | 67mm W x 68mm H x 24.1mm D (2.65” x 2.7” x 0.95”) |
| SENSOR APERTURE | 20.3mm (0.8”) |
| ENCLOSURE MATERIAL AGENCY | ABS/PC, UL94-V0 |
| APPROVALS | cULus listed |