WATER DETECTOR – TWO CHANNELS WITH LOCAL SPOT AND REMOTE

WLD2S Series

The WLD2S sensor has two detection channels.  The first is integrated gold-plated probes for detecting conductive liquids.  The second channel is a remote conductive cable available with different lengths. Height adjustable legs are standard.  Tri-color LED viewable LED provided for status.  The alarm contacts are fail-safe, reporting if water detection or power failure occurs.

PRODUCT HIGHLIGHTS

- monitors variety of conductive liquids

- alarm on liquid detection, power failure, or internal failure

- dual liquid detection channels

- Various length probes available

- gold plated spot detector

- conductive cable

- fail-safe circuitry

- NO/NC form C, relay output

- weatherproof enclosure

- adjustable legs for adjusting detection height

SPECIFICATIONS

| SPECIFICATIONS |  |
| --- | --- |
| POWER SUPPLY | 14 - 30 Vac/dc |
| SUPPLY CURRENT | 125 mA max @ 24 Vdc, (no water) |
| OPERATING TEMPERATURE | 0 to 50°C (32 to 122°F) |
| ENCLOSURE | ABS with hinged lid and gasket, UL94 V-0, IP65 (NEMA 4X) |
| DIMENSIONS | 145mm L x 100mm W x 64mm H (5.7” x 3.95” x 2.5”) |
| HEIGHT WITH MOUNTING | 98.43mm (3.875”) maximum |
| LEGS (WD100) | 88.90mm (3.25”) minimum |
| ALARM OUTPUT | Form C relay, rated @ 2 Amps @ 250 Vac/dc (resistive load) |
| APPROVALS | CE, RoHS |
| COUNTRY OF ORIGIN | Canada |